VPA1407120 Series Module Data Sheet

Specification

| Test items |  | Indicator | Remark |
| --- | --- | --- | --- |
| Operating Voltage |  | 28V | 28-30V |
| Maximum Output Power |  | 47±0.5dBm |  |
| Operating Current |  | ≤5A | @50W Typical |
| Gain |  | 47±1.0dB |  |
| In-band Fluctuation |  | ±1.5dB | Peak-to-peak value |
| Maximum Allowable Input Power |  | ≤10dBm | Lasts 1min without damage |
| Input VSWR |  | ≤1.50 | Add +28V, standard network output -10dBm |
| Output VSWR |  | ≤1.50 | No power applied, standard network output -10dBm |
|  |  | ≤1.50 | Power applied,bidirectional coupler testing |
| Temperature Test | Operating Environment Temperature | -10～+55℃ | Low-temperature startup, monitoring functions properly |
|  | Gain Stability | ±1.5dB@-10℃～+55℃ |  |
|  | Power Stability | ±1.5dB@-10℃～+55℃ |  |
| Power Requirements |  | ≥4.5A @+28Vdc； | Continuous wave output of 50W |
| Power Connector |  | Feedthrough Capacitor core: VCC+28V; Ground plate: GND |  |
| Function of 5Pin XH-2.54mm |  | 1.TEMP_DET: Temperature detection AD voltage value 2.PA_FWD: Forward power detection AD voltage value 3.PA_REV: Standing wave alarm, high-level alarm, low-level elimination 4.PA_EN: Enable switch; open with high-level 3V~5V, close with low-level<2V.(For installation, please confirm with sales.) 5.PA_ADJ: NC |  |
| RF output connector |  | SMA-F |  |
| Module Protection |  | Over-temperature protection; VSWR protection |  |
| Signal Source Input |  | Built-in high-speed noise modulation signal source |  |
| LED Module Indicators |  | POWER: Power indicator； RUN: Operation indicator； VSWR: VSWR alarm indicator |  |
| Size |  | 140*71*21mm |  |
| Weight |  | 390g |  |
| Packing Size |  | 210*108*38mm |  |
| Packing Weight |  | 445g |  |

Produce Range

| Supported Frequency Range (MHz) | Max Bandwidth (MHz) | Max Power (W) |
| --- | --- | --- |
| 400-700 | 300 | 50W |
| 700-1000 | 300 | 100W |
| 1100-1200 | 100 | 80W |
| 1200-1500 | 300 | 50W |
| 1500-1600 | 100 | 100W |
| 1600-4000 | 300 | 50W |
| 4000-5800 | 300 | 45W |
| 5800-6000 | 200 | 40W |

Note: The "Supported Frequency Range (MHz)" in this table is not the actual interference frequency band of a single module. It is the production range supported by the module. The actual interference frequency band of the module is included in the "Supported Frequency Range (MHz)" and is limited by the "Maximum Bandwidth (MHz)".

Dimensions

Frequency＜4800MHz :

Frequency＞4800MHz :