Pine-INT-5

Large Sphere 5-Band Detection & Jamming Integrated Quad Array

| Fixed – Detection Large Sphere + 5-Band Jamming | Vehicle-Mounted – Detection Large Sphere + 5-Band Jamming |
| --- | --- |

Product Introduction

The 5-band detection and jamming integrated quad array system consists of a detector and a four-sided directional jamming array, capable of detecting, identifying, and jamming UAVs. It can cause UAVs to hover in place, force-land, or return to home, effectively protecting low-altitude airspace safety and preventing UAVs from entering the defense zone. It features excellent detection and defense performance, long-distance operation capability, requires no personnel on duty, and is capable of automatic UAV detection and defense. It can be deployed in fixed or vehicle-mounted configurations according to different scenario requirements. Direction-finding functionality is also available as an option.

Features

Omnidirectional Transmission: Four-sided directional jamming devices form a 360° omnidirectional jamming enclosure.

Networked Management: Multiple systems can be networked and centrally managed via LAN/Internet.

Unattended Operation: Automatic response strategies can be configured in the software backend for 24-hour unattended operation.

Product Specifications

|  | Parameter Name | Parameter Value |
| --- | --- | --- |
| System | Overall Dimensions | 700 × 700 × 445 mm (excluding tripod and vehicle-mounted base) |
|  | Overall Weight | 26 kg |
|  | Deployment Mode | Fixed, vehicle-mounted |
|  | Power Supply | AC 220 V mains power |
|  | Power Consumption | Directional jamming chassis: 1120 W (full load operation) |
|  | Operating Temperature | -20 ~ 50 ℃ |
| Detection (large sphere) | Detection Range (radius) | 1 ~ 10 km+ (varies with environment and UAV model) |
|  | Detection Frequency | 2.4 GHz, 5.2 GHz, 5.8 GHz, 0.4 GHz, 0.9 GHz, 1.4 GHz |
|  | Detection Angle | Omnidirectional 360° |
|  | Direction Finding Accuracy | ≤ 3° (RMS) |
|  | Positioning Accuracy | ≤10m(RMS) |
| Jamming | Jamming Frequency | 800 M, 900 M, 2.4 G, 5.2 G, 5.8 G |
|  | Jamming Power | 50 W (800 M), 50 W (900 M), 65 W (2.4 G), 50 W (5.2 G), 65 W (5.8 G) |
|  | Jamming Range | 3 km (in open line-of-sight environment) |
|  | Jamming Angle | Horizontal: 65°, Vertical: 30° (directional) |
|  | Protection Functions | Temperature control protection, reflection protection |