VPA1629226 Module Data Sheet

- Radio Frequency Indicators

| Test items |  |  | Indicator | Remark |
| --- | --- | --- | --- | --- |
| Operating Voltage |  |  | - 28V | - 28-30V |
| Maximum Output Power |  |  | - 50±0.5dBm |  |
| Operating Current |  |  | - 9A | - @100W Typical |
| Gain |  |  | - 50±1.0dB |  |
| In-band Fluctuation |  |  | - ±2dB | - Peak-to-peak value |
| Maximum Allowable Input Power |  |  | - ≤10dBm | - Lasts 1min without damage |
| Input VSWR |  |  | - ≤1.50 | - Add +28V, standard network output -10dBm |
| Output VSWR |  |  | - ≤1.50 | - No power applied, standard network output -10dBm |
|  |  |  | - ≤1.50 | - Power applied,bidirectional coupler testing |
| Temperature Test | Operating Environment Temperature (°C) |  | - -10～+55℃ | - Low-temperature startup, monitoring functions properly |
|  | Gain Stability |  | - ±1.5dB@-10℃～+55℃ |  |
|  | Power Stability |  | - ±1.5dB@-10℃～+55℃ |  |
| Power Requirements |  |  | - ≥9A @+28Vdc； | - Continuous wave output of 100W |
| Power supply, control interface 7W2 |  | - A1: VCC+28V; A2: GND - 1: TEMP_AL: Temperature detection AD voltage - 2: RUN: Forward power detection AD voltage - 3: VSWR_AL: VSWR alarm, high level for alarm, low level for normal operation - 4: PA_EN: Enable switch; high level 3V5V to turn on, low level <2V to turn off - 5: PA_ADJ: NC |  |  |
| RF output connector |  |  | - N-F |  |
| Module Protection |  |  | - Temperature compensation； - Over-temperature protection； - VSWR protection |  |
| Signal Source Input |  |  | - Built-in high-speed noise modulation signal source |  |
| LED Module Indicators |  |  | POWER: Power indicator；RUN: Power operation indicator；VSWR: VSWR alarm indicator |  |

- Produce Range

| Supported Frequency Range (MHz) | Max Bandwidth (MHz) | Max Power (W) |
| --- | --- | --- |
| 700-830 | 130 | 150W |
| 830-1000 | 170 | 200W |
| 1000-1300 | 300 | 100W |
| 1300-1400 | 100 | 100W |
| 1500-1600 | 100 | 150W |
| 1800-1900 | 100 | 150W |
| 2000-2200 | 200 | 100W |
| 2400-2500 | 100 | 200W |
| 3300-3600 | 300 | 50W |
| 4800-6000 | 400 | 80W |

Note: The "Supported Frequency Range (MHz)" in this table is not the actual interference frequency band of a single module. It is the production range supported by the module. The actual interference frequency band of the module is included in the "Supported Frequency Range (MHz)" and is limited by the "Maximum Bandwidth (MHz)".

- Dimensions