Forger-CM Introduction and Operation Guide

V1.0

- ⚠Risk warning:

- It is forbidden to use the device outdoors in thunderstorms;

- It is forbidden to put the device near fire;

- It is forbidden to disassemble the device when it is powered on;

- Avoid the device from falling into water or liquid flowing into it;

- Avoid serious collisions with the device to prevent the internal lithium battery from burning;

- Before turning on the device, check whether the antenna has been fully connected to the corresponding interface;

- When the device is turned on, patients wearing implantable electronic therapeutic devices (such as pacemakers) should strictly keep a distance of more than 10 meters from the device

- Specification

| *Channels | Frequency(MHz) |  | Output Power |
| --- | --- | --- | --- |
| CH1 | 420-480 |  | 50W |
| CH2 | 860-930 |  | 50W |
| CH3 | 1160-1300 |  | 50W |
| CH4 | 1300-1400 |  | 50W |
| CH5 | 1520-1620 |  | 50W |
| CH6 | 2400-2483.5 |  | 100W |
| CH7 | 5150-5350 |  | 80W |
| CH8 | 5720-5850 |  | 80W |
| Jamming Distance |  | 3km |  |
| Weight |  | 26.5kg（without antenna） |  |
| Size |  | 47*38*28cm（without antenna） |  |
| Power Supply |  | AC 210V~240V |  |
| Temperature/humidity |  | -20℃-+50℃/≤80% |  |
| Machine reliability |  | MTBF not less than 1000 hours |  |
| System response time |  | No more than 30 seconds (from power-on to normal system operation) |  |

*Note: This table is a typical interference frequency table for 8-channel equipment. The number of channels and interference parameters of the actual finished product need to be confirmed after communication with the customer.

- Connection

- The device has a built-in switch, and users can choose one of the two RJ45 sockets to connect the detection device and use the other socket to connect the computer.

- How to modify the IP address of the device:

- Open USR-MO software. (The software version in the picture is for 	demonstration only, please refer to the actual version sent)

- Change the IP address in the following order:

- 1) Click "Search Device";

- 2) Click the device in the search results bar;

- 3) Modify the static IP address of the device;

- 4) Modify the device gateway;

- 5) Click Save.

- CAD of Size

- Pole Mounting Example

- Installation structure diagram

- Installation completed diagram