WBPA17011025-100W

- Operating frequency Bands

| Frequency(MHz) | Output Power(W) | Current | Transistor |
| --- | --- | --- | --- |
| 2500-5000 | 100W | ≤10A | GaN |

- RF Specifications

| Test Items | Indicator | Remark |
| --- | --- | --- |
| Operating Voltage | 28V | 24-30V |
| In-band Fluctuation | ≤±2dB | Peak-to-peak Value |
| Output VSWR | ≤2.50 | Power-on, dual-directional coupler test |
| Operating Temperature | -20～+65℃ |  |
| Power Supply / Communication Interface：7W2 | A1:GND 1pin:485A 2pin:485B 3pin:GND 4pin:SW EN switch: high level (3.3V-5V) enables 5pin:NC A2:GND |  |
| RF Output Connector | N-F |  |
| Module protection | temperature compensation, over-temperature protection, VSWR protection |  |
| LED status indicator | POWER: Power indicator RUN: Operating status indicator ALARM: VSWR alarm indicator |  |

Software settings

| Monitoring function |  | Description | Remarks |
| --- | --- | --- | --- |
| Power amplifier settings parameters | Power amplifier switch | ON/OFF |  |
|  | Alarm switch | Enable or disable alarm function |  |
|  | Downlink power control | Set power amplifier output power (adjustment range ≥ 20 dB) |  |
|  | Power Amplifier Over-Temperature Alarm | Configurable PA high-temperature shutdown threshold: shuts down at >85°C, recovers at 65°C. |  |
|  | Power Amplifier VSWR Alarm | VSWR > 4: Power amplifier auto shutdown |  |
|  | Power Amplifier Overcurrent Alarm | Current > 12 A: Power amplifier auto shutdown |  |
|  | Power amplifier underpower alarm | Power < 33 dBm: PA underpower alarm |  |
|  | Power amplifier overvoltage alarm | Voltage > 32 V: PA auto shutdown |  |
|  | Power Amplifier Undervoltage Alarm | Voltage < 20 V: PA undervoltage alarm |  |
| Power Amplifier Parameter | Power Amplifier On/Off Status | If the power amplifier is set to “OFF”, it is in the OFF state; otherwise, it is in the ON state. |  |
|  | Downlink VSWR | 1.2 under normal conditions; 4.0 under mismatched load conditions. |  |
|  | Temperature Query | Internal module temperature (range: -40°C to +100°C) | Measurement error: ±3°C |
|  | Voltage Detection | Detection Module Operating Voltage |  |
|  | Current Detection | Detection Module Operating Current |  |