DPA15712328 DDS Module Data Sheet

Introduction

- This module is developed based on SDR (Software-defined Radio) technology, generating and amplifying a maximum bandwidth of about 400MHz between 20MHz and 6GHz.

- Application range: Signal jammer for wireless communication products such as mobile phones/walkie-talkies/RC Drone/RC Car/RC Boat.

Produce Range

| Selectable Frequency Range (MHz) | Maximum selectable bandwidth (MHz) | Maximum selectable output power (W) |
| --- | --- | --- |
| 20-100 | 80 | 100 |
| 100-500 | 250 | 100 |
| 700-1000 | 300 | 100 |
| 1000-1550 | 300 | 70 |
| 1550-1990 | 300 | 100 |
| 2010-2700 | 300 | 100 |
| 3300-3600 | 300 | 80 |
| 4800-6000 | 300 | 80 |

Note: The "Selectable Frequency Range" shown in this table refers to the manufacturing coverage, not the exact output range of a single module. Each module must operate within this range, with an actual output bandwidth that does not exceed the “Maximum Selectable Bandwidth.”

- Radio Frequency Indicators

| Test items |  |  | Indicator | Remark |
| --- | --- | --- | --- | --- |
| Operating Voltage |  |  | 28V | 28-30V |
| Conversion Efficiency |  |  | ≥42% | @100W Typical |
| Controllable Output Power |  |  | 10W～100W | Software-configurable power |
| ALC Power Adjustment Range |  |  | ≥30dB | Voltage-Controlled Attenuation |
| Gain |  |  | 50±1.0dB |  |
| In-band Fluctuation |  |  | ≤2.5dB | Peak-to-peak value |
| Spurious Emission | In-band |  | ≤-15dBm/1MHz | Measure at the center frequency with CW signal at maximum output power |
|  | Out-of-band | 9KHz～1GHz | No higher than normal background noise and interference |  |
|  |  | 1G～12.75GHz |  |  |
| Maximum Allowable Input Power |  |  | ≥+10dBm | Lasts 1min without damage |
| Input VSWR |  |  | ≤1.35 | Add +28V, standard network output -10dBm |
| Output VSWR |  |  | ≤1.30 | No power applied, standard network output -10dBm |
|  |  |  | ≤1.35 | Power applied, bidirectional coupler testing |
| Temperature Test |  | Operating Environment Temperature | -10～+65℃ | Low-temperature startup, monitoring functions properly |
|  |  | Gain Stability | ±1.5dB @-40℃～+65℃ |  |
|  |  | Power Stability | ±1dB @-40℃～+65℃ |  |
| Power Requirements |  |  | ≥8A @+28Vdc； | Continuous wave output of 100W |
| Power interface, communication interface RS485 |  |  | 7W2 | Male connector |
| RF Output Connector |  |  | N-F |  |
| LCD Liquid Crystal Display |  |  | Display frequency, power, and on/off status. |  |
| LED Module Indicators |  |  | POWER: Power indicator，RUN: Power operation indicator，ALARM: Alarm indicator |  |
| Built-in cooling fan |  |  | The module can run independently. |  |
| Size |  |  | 157*123*28mm |  |
| Weight |  |  | 863g |  |

- Module Software Configuration

| Monitoring Function |  | Description | Remark |
| --- | --- | --- | --- |
| Amplifier Configuration Parameters | Amplifier Switch | Turn the amplifier on or off. |  |
|  | Alarm Switch | Enable or disable the alarm function. |  |
|  | Downlink Power Control | Adjustable maximum output power of the amplifier, with an adjustable range of ≥20dB, and the ability to set high, medium, and low power levels. |  |
|  | Amplifier Over-Temperature Alarm | It will automatically shut down when the temperature exceeds 85°C and automatically resume operation when the temperature drops to 65°C |  |
|  | Amplifier Over-Power Alarm | The system will be automatically shut down when the actual power value is 2dB higher than the downlink ALC control threshold. |  |
|  | Amplifier Standing Wave Alarm | Automatically shut down when the standing wave ratio is greater than 4 |  |
|  | Amplifier Over-Current Alarm | Automatically shut down when current exceeds 12A |  |
|  | Amplifier Under-Power Alarm | When the power is less than 37dBm, an under-power alarm is displayed. |  |
|  | Amplifier Over-Voltage Alarm | If the amplifier voltage exceeds 32V, the amplifier will automatically shut down. |  |
|  | Amplifier Under-Voltage Alarm | If the amplifier voltage is less than 20V, it will display an under-voltage alarm. |  |
| Amplifier Query Parameters | Amplifier Switch Status | If the amplifier is set to "OFF", the corresponding query should indicate that the amplifier is in the "OFF" state. Conversely, if the amplifier is set to "ON", the query should indicate that the amplifier is in the "ON" state. |  |
|  | Downlink Output Power Level | Detect the output power value of the amplifier within the detection range of (Pomax+2, Pomax-20). | Detection accuracy: Pout±1dB. |
|  | Downlink Standing Wave Ratio | The normal standing wave ratio is 1.2. When there is a mismatched load, the query value will be 4.0. |  |
|  | Temperature Query | Detect the internal temperature of the module within a detection range of -40℃ to +100℃. | Detection error: ±3°C. |
|  | ALC Value Query | The displayed value is equal to the actual set ALC value. |  |
|  | Voltage Detection | Detecting module operating voltage. |  |
|  | Current Detection | Detecting module operating current. |  |
| Signal Source Configuration | Frequency Setting Mode 1 | Setting arbitrary single frequency point within the range. | Frequency setting accuracy of 1MHz, maximum frequency bandwidth of 500MHz, and highest frequency of 6GHz. |
|  | Frequency Setting Mode 2 | Configure wideband sweep mode, frequency, interval, step size, and other parameters. |  |
|  | Frequency Setting Mode 3 | Set any 4 sub-frequency segments within the bandwidth, with the sub-segments incrementing. |  |
| Module Communication Protocol |  | RS485 |  |
| Interface Definition |  | +:28V, -: GND; Control bit 1:485A, 2:485B. |  |

Dimensions

Single module software control instructions

- Power setting adjustment, the left side displays the power value in dBm and W units respectively, and the right scroll bar can be dragged left and right to adjust the power.

- The default frequency range of the first channel, Start Freq is the starting frequency, Stop Freq is the ending frequency, and the ending frequency must be greater than the starting frequency. The default frequency step is 1M, which can be adjusted accurately.

- The frequency range setting of the second channel, the starting frequency of the second channel must be greater than the ending frequency of the first channel.

- The frequency range setting of the third channel, the starting frequency of the third channel must be greater than the ending frequency of the second channel.

- The function is the same as above, which is the frequency range setting of the fourth channel.

- The power amplifier module switch setting. ON is turned on, and OFF is turned off the power amplifier.

- Working status display:

Out Power: output power, the real-time power of the current module when it is working 			normally.

Temperature: module temperature.

Voltage: module supply voltage.

Current: module current.

VSWR: standing wave state.

8. Serial port settings, RS485 communication interface. Select the serial port number in the drop-down list box, Open to open, Close to close.

9. Frequency channel selection box, a total of 4 channels are available.

10. Module address code query and setting.

11. Query or set parameters, Inquiry is to query parameters, Set is to apply all modified parameters.

12. Display information feedback when querying or setting parameters.

Sub-band setting effect display

DDS RS485 communication connection diagram