Lance-CM Drone Jammer Gun

Instruction Manual

Product images:

Front

Back

Introduction:

The anti-drone gun emits electromagnetic waves through multiple specific frequency bands to interfere with the signal transmission system of commercial drones, thereby achieving drone countermeasures. The product has a simple design and is easy to operate, which is convenient for front-line personnel to use and train. It is suitable for multiple low-altitude security scenarios such as airports, stations, petrochemical oil depots, border inspections, and gatherings.

Features:

Directional emission: Directed emission of radio blocking signals with concentrated energy;

Flexible and convenient: Portable design, light equipment, simple operation, high maneuverability;

Strong endurance: Built-in high-quality battery, long service life;

Multi-band countermeasure: Can counter the communication, image transmission, and navigation links of various drones.

- ⚠Risk Warning

- When the equipment is deployed outdoors, ensure that it is within the effective range of the lightning protection facilities;

- It is strictly forbidden to keep the equipment close to fire sources

- It is strictly forbidden to disassemble the equipment when it is powered on.

- Avoid dropping the device into water or allowing liquid to enter the device.

- Before turning on the device, check whether the antenna is fully connected to the corresponding interface.

- When the device is turned on, patients wearing implantable electronic therapeutic devices (such as pacemakers) should strictly keep a distance of more than 10 meters from the device.

- Packing List

| Item | Quantity | Unit |
| --- | --- | --- |
| Jammer gun | 1 | piece |
| Battery | 2 | piece |
| Charger | 1 | piece |
| Baldric | 1 | piece |
| Aviation box key | 1 | piece |

- Jamming Frequency Band:

- (power and frequency can be customized according to needs)

| Channels | Frequency | Power |
| --- | --- | --- |
| CH1 | 430-440MHz | 25W |
| CH2 | 860-930MHz | 25W |
| CH3 | 1170-1280MHz | 25W |
| CH4 | 1430-1444MHz | 25W |
| CH5 | 1550-1620MHz | 25W |
| CH6 | 2400-2485 MHz | 25W |
| CH7 | 5150-5350MHz | 25W |
| CH8 | 5725-5850MHz | 25W |

- Specifications:

| Frequency and power | 433M~5.8G/200W (customizable according to customer needs) |
| --- | --- |
| Antenna direction | Built-in high-gain directional antenna |
| Antenna gain | 5.8G>12dBi, 2.4G>10dBi, 1.5G>8dBi, 1.2G>8dBi |
| System protection | Output standing wave protection, temperature protection |
| Appearance | Handheld gun type |
| Cooling system | Built-in intelligent cooling system |
| Control direction | Directional 60 degrees |
| Control distance | >1 km |
| Battery | Built-in 10A (28V) battery |
| Charging method | External 4A charger, turn on the external switch to charge, automatically manage the charging current, and automatically disconnect when full |
| RF output control | 1 main power switch, 8 independent frequency band control switches |
| Power supply method | Built-in DC-28V battery power supply, sustainable power supply time>1h |
| Battery power display | Built-in battery indicator light at the buttstock |
| Sight | High-precision sight |
| Total weight (including antenna) | About 4.6kg |
| Operating temperature | -35°C～+70°C |
| Operating humidity | 80% |

CAD of the Size: